# Data Analysis (Train: trainset_synth, Test: feb2026rb2r2)

The data analysis is based upon the predictions.csv files generated by 3 models viz. DNN, DeepNN and LSTM for voltage and power respectively.

Summary: Across all models, prediction error is consistently driven by load_current and current, showing strong positive correlation, especially for DNN and DeepNN. Feedforward models (DNN, DeepNN) exhibit high errors and struggle with nonlinear behavior and extreme load conditions, particularly in power prediction. In contrast, LSTM significantly reduces errors and shows stable performance, indicating effective capture of temporal dynamics and system behavior. Overall, model performance improves from DNN to DeepNN to LSTM, with LSTM being the most reliable across both voltage and power tasks.

Test Data Distribution:

| current | heater_power | load_current | eclipse_flag | payload_status |
| --- | --- | --- | --- | --- |
| Mean: 3.8349 | Mean: 215.1674 | Mean: 9.8233 | Mean: 0.7486 | Mean: 0.2266 |
| Std : 5.5261 | Std : 87.3810 | Std : 3.5540 | Std : 0.4338 | Std : 0.4187 |

Range:

| current | heater_power | load_current | eclipse_flag | payload_status |
| --- | --- | --- | --- | --- |
| -0.0531 → 87.3413 | 10.8000 → 446.8000 | 4.1735 → 88.8946 | 0.0000 →1.0000 | 0.0000 → 1.0000 |

1. Voltage DNN:

Feature vs Error Correlation:

current : 0.3772

heater_power : -0.1002

load_current : 0.7883

eclipse_flag : -0.0228

payload_status : 0.0384

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 21.7590 |
| heater_power | 17.6375 |
| load_current | 16.6674 |
| eclipse_flag | 17.0260 |
| payload_status | 17.0260 |

The model’s error shows a strong positive correlation with load_current (0.78), indicating that prediction accuracy degrades as load increases. Moderate correlation with current (0.37) further suggests sensitivity to operating conditions. Error-by-bin analysis reveals the highest error in high current regions (~21.7), confirming poor handling of extreme values. Overall, the model struggles with nonlinear behavior and skewed data containing rare high-load spikes.

2. Voltage DeepNN:

Feature vs Error Correlation:

current : 0.3844

heater_power : -0.0687

load_current : 0.8219

eclipse_flag : -0.0189

payload_status : 0.0706

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 18.9955 |
| heater_power | 13.6817 |
| load_current | 13.8490 |
| eclipse_flag | 13.3008 |
| payload_status | 13.3008 |

The DeepNN model shows a strong positive correlation with load_current (0.82) and moderate correlation with current (0.38), indicating error increases with higher load conditions. Compared to DNN, overall errors are reduced (~13–19), showing improved learning. However, the highest error still occurs in high current regions (~19), indicating continued difficulty in handling extreme values. Thus, while DeepNN improves accuracy, it still struggles with nonlinear behavior under high-load conditions.

3. Voltage LSTM:

Feature vs Error Correlation:

current : 0.1469

heater_power : -0.2668

load_current : 0.2291

eclipse_flag : -0.0238

payload_status : 0.0286

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 1.4725 |
| heater_power | 1.5660 |
| load_current | 1.5058 |
| eclipse_flag | 1.5386 |
| payload_status | 1.5386 |

The LSTM model shows weak positive correlation with load_current (0.23) and current (0.15), indicating minimal error dependence on load conditions. Negative correlation with heater_power (-0.27) suggests slight stabilization in certain regimes. Error values are very low (~1.47–1.56) across all features, showing consistent performance. Overall, LSTM effectively captures temporal patterns and handles nonlinear behavior, significantly outperforming DNN and DeepNN.

4. Power DNN:

Feature vs Error Correlation:

current : 0.3614

heater_power : -0.0757

load_current : 0.8107

eclipse_flag : 0.0069

payload_status : 0.0589

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 191.5049 |
| heater_power | 171.6738 |
| load_current | 168.7202 |
| eclipse_flag | 155.2454 |
| payload_status | 155.2454 |

The Power DNN model shows strong positive correlation with load_current (0.81) and moderate correlation with current (0.36), indicating error increases significantly with load conditions. Extremely high bin errors (~155–191) reveal poor prediction accuracy, especially in high current regions. This indicates severe difficulty in capturing power dynamics (V×I relationship) under varying loads. Overall, the model struggles with nonlinear interactions and error propagation in power prediction.

5. Power DeepNN:

Feature vs Error Correlation:

current : 0.3885

heater_power : -0.1164

load_current : 0.7677

eclipse_flag : -0.0735

payload_status : 0.0068

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 191.6556 |
| heater_power | 176.6691 |
| load_current | 136.7308 |
| eclipse_flag | 122.8142 |
| payload_status | 122.8142 |

The Power DeepNN model shows strong positive correlation with load_current (0.77) and moderate correlation with current (0.39), indicating error increases with load intensity. While slightly improved over DNN in some regions (lower error for load_current), overall errors remain very high (~122–191). The highest error still occurs in high current regions, showing difficulty in modeling power under extreme conditions. Thus, DeepNN offers limited improvement and still struggles with nonlinear power interactions and load-driven variability.

6. Power LSTM:

Feature vs Error Correlation:

current : 0.3681

heater_power : -0.1753

load_current : 0.7243

eclipse_flag : -0.0254

payload_status : 0.0862

Error by feature bins:

| Feature | Average Error |
| --- | --- |
| current | 29.7506 |
| heater_power | 20.2057 |
| load_current | 38.7372 |
| eclipse_flag | 18.4903 |
| payload_status | 18.4903 |

The Power LSTM model shows strong positive correlation with load_current (0.72) and moderate correlation with current (0.37), indicating error still increases with load. However, errors are significantly reduced (~18–39) compared to DNN/DeepNN, showing much better performance. The highest error occurs in load_current (~38.7), indicating some difficulty in extreme load regions. Overall, LSTM effectively captures temporal and nonlinear power behavior, though minor load-driven errors remain.